Digital Maturity Assessment (DMA) Report

Factory: Wafer (Potato Chips) Production Unit
Date of Assessment: [Insert Date]
Prepared by: Dr. Vaibhav Khavkar, Sneha Khavkar, Prerana Kulkarni

1. Introduction

The wafer factory is a mid-scale food processing unit producing approximately 10,000 kg/day of potato chips. The facility operates a semi-automated production line, with manual interventions in critical stages such as peeling, cleaning, and quality inspection.

Purpose of Assessment:
To evaluate the digital readiness of the factory across process automation, data management, quality monitoring, equipment integration, workforce digital skills, and sustainability, identifying opportunities for digital transformation.

2. Methods

- Direct Observation: Walkthrough of 11 production stages noting data generation and automation.

- Interviews: Discussions with line operators, supervisors, and QA staff.

- Document Review: Production logs, batch sheets, maintenance records.

- Photo & Video Documentation: Captured key stages.

3. Scale for Scoring

| Score | Meaning |
| --- | --- |
| 1 | Fully Manual / No Data Logged |
| 2 | Basic Mechanization / Manual Recording |
| 3 | Partial Automation / Limited Digital Data |
| 4 | Mostly Automated / Digitally Integrated |
| 5 | Fully Automated / Real-Time Connected Data |

4. Factory Process Overview

This table summarizes all key production stages from raw potato intake to final packing. It lists inputs, activities, outputs, and highlights where data is generated and potential areas for digitalization. This helps identify stages with high automation potential and critical points where data capture is currently limited.

| Process No. | Input / Resources | Activity | Output | Data Generation | Digital Potential |
| --- | --- | --- | --- | --- | --- |
| 1 | Potatoes | Weighing | Measured potatoes | Yes: weight, batch ID | IoT-enabled weighing scale |
| 2 | Potatoes | Quality Check (Raw) | Quality report | Yes: defect images, grade | AI vision system |
| 3 | Potatoes, water | Cleaning | Cleaned potatoes | Partially | Sensor-based water flow, automated line |
| 4 | Potatoes | Peeling | Peeled potatoes | Partially | Automated peeling machine |
| 5 | Peeled potatoes | Slicing | Thin potato slices | Yes: slice thickness, speed | Sensor-monitored slicer |
| 6 | Slices, water | Cleaning (Slices) | Cleaned slices | Partially | Flow & temperature sensors |
| 7 | Cleaned slices, oil | Frying | Fried chips | Yes: temp, frying time, batch logs | PLC-controlled fryer |
| 8 | Fried chips, spices | Flavoring | Flavored chips | Yes: seasoning dosage | Automated flavoring system |
| 9 | Chips | Quality Control | Perfect quality chips | Yes: defect count, camera logs | AI vision inspection |
| 10 | Chips | Taste Balancing | Uniform taste | Partially | Flavor calibration system |
| 11 | Chips | Packing | Packed chips | Yes: package count, batch ID | Automated packaging, nitrogen flushing |

5. DMA Evaluation Table

This table evaluates the factory against the six DMA pillars — Process Automation, Data Management, Quality Monitoring, Equipment Integration, Workforce Skill & Digital Culture, and Sustainability. Each row contains observation questions, answers based on actual process observations, evidence sources, scores on a 1–5 scale, and comments/feedback. It provides a detailed, evidence-based assessment of digital maturity.

| Section (Pillar) | Observation Questions | Answers (Observed) | Data Source | Score | Comments |
| --- | --- | --- | --- | --- | --- |
| Process Automation | Steps manually controlled | Peeling & Cleaning | Observation | 3 | Frying, weighing automated; peeling & cleaning manual |
|  | Sensors/PLCs/timers | Fryer, slicer, packaging | Machine inspection | 4 | Core machines instrumented; no IoT dashboard |
|  | Auto data logged | Temp, weight, packaging count | HMI/PLC logs | 3 | Limited central storage |
|  | Operator manual adjustment | 2–3 batches per shift | Observation | 2 | Manual adjustments cause inconsistency |
| Data Management | How recorded | Paper logs, Excel | Production records | 2 | No centralized database |
|  | Batch linked to parameters | No | Batch sheet review | 1 | Traceability missing |
|  | Data entry/verification | Supervisor manually | QA records | 2 | Delayed entry, error-prone |
|  | Frequency reviewed | Weekly | QA meetings | 3 | Data reviewed but not analyzed |
| Quality Monitoring | Quality check method | Visual inspection + taste | QA observation | 2 | No real-time sensor monitoring |
|  | Real-time logging | Post-production | QA sheets | 2 | Only after production |
|  | Good quality definition | Color, crispness, breakage | Checklist | 3 | Defined but not digitized |
|  | Action on defects | Manual rework | Operator interview | 2 | No automated rejection |
| Equipment Integration | Machines networked? | Independent lines | Machine check | 2 | No SCADA or dashboard |
|  | Data transfer | USB/manual copy | Maintenance | 2 | Risk of data loss |
|  | Downtime/alarms | Temp alarms only | Panel inspection | 3 | Partial alerting |
| Workforce Skill | Trained on digital tools | Few operators know HMI | Training records | 2 | Need digital upskilling |
|  | Use data for decisions | Mostly experience-based | Observation | 2 | Low data-driven culture |
|  | Feedback collected | No formal system | Staff interviews | 1 | Missed opportunity |
| Sustainability | Resource tracking | Meter readings daily | Utility logs | 2 | Manual tracking |
|  | Recycling/reuse | Oil filtered once | Maintenance | 3 | Partial implementation |
|  | Waste/rejects recorded | Weight logged manually | QA logs | 2 | No automated scrap monitoring |
|  | Threshold alerts | None | System review | 1 | No alert system |

6. Summary of DMA Scores

This table presents the average scores per DMA pillar, their corresponding maturity levels, and key remarks. It provides a quick overview of the factory’s overall digital maturity and highlights the most significant gaps and opportunities for improvement.

| Pillar | Average Score | Maturity Level | Remarks |
| --- | --- | --- | --- |
| Process Automation | 3.0 | Partially Automated | Critical areas still manual |
| Data Management | 2.0 | Developing | Manual logs, limited integration |
| Quality Monitoring | 2.2 | Developing | Mostly manual inspections |
| Equipment Integration | 2.2 | Developing | No SCADA, low interconnectivity |
| Workforce Skill | 2.0 | Developing | Upskilling needed |
| Sustainability | 2.1 | Developing | Manual tracking; no real-time monitoring |

Overall Digital Maturity Score: 2.25 / 5 → Developing Stage

7. Observed Gaps & Opportunities

- Introduce IoT-enabled automation at peeling and cleaning stages.

- Implement central ERP or MES for batch tracking and digital logs.

- Deploy AI vision-based QC for real-time defect detection.

- Install SCADA/dashboard for real-time monitoring.

- Train workforce on digital tools and data-driven decision making.

- Add IoT-enabled meters for energy, water, and oil; automate threshold alerts.

8. Recommended Next Steps

- Conduct pilot automation project for peeling and slicing.

- Install IoT sensors on critical machines for real-time data logging.

- Deploy centralized dashboard/SCADA system.

- Launch training program for operators.

- Implement continuous monitoring of sustainability KPIs.