- Introduction

The factory is a mid-scale wafer (snack) production unit operating with semi-automated systems. It produces 10,000 kg/day of potato-based wafers. Current challenges include manual data logging, inconsistent production quality tracking, and limited automation in monitoring processes.

The purpose of this DMA is to evaluate the digital readiness of the wafer factory across key operational areas — production, quality, maintenance, and supply chain — and to identify digital transformation opportunities that can enhance efficiency, traceability, and quality consistency.

- Methods

The assessment team conducted on-site observations, staff interviews, and photographic documentation of the raw material and frying processes. Line supervisors, production engineers, and machine operators were interviewed. Workflow videos were reviewed to understand machine interconnectivity and control systems.

- Scale

A 1–5 scale was used, where 1 represents “Manual / Initial Stage” and 5 represents “Fully Digitally Integrated.”

- Process-Wise DMA Table

| - Process Area | Pillar | Observations | Score (1–5) | Comments / Recommendations |
| --- | --- | --- | --- | --- |
| Raw Material Preparation | Process Automation | Sorting & washing machines are semi-automatic; manual inspection of potatoes still required. | 3 | Introduce AI-based defect detection for potato grading and automate washing line sensors. |
|  | Data Management | Batch data recorded manually in notebooks. | 2 | Implement barcode/RFID tracking for batches and digital data entry systems. |
|  | Quality Monitoring | No real-time moisture or contamination sensors. | 2 | Add IoT-based quality sensors during washing and slicing. |
|  | Equipment Integration | Machines operate independently without central control. | 2 | Introduce PLC and SCADA systems for centralized control and monitoring. |
|  | Workforce Skill | Staff trained for machine operation but not data entry. | 3 | Conduct digital literacy training. |
|  | Sustainability | High water consumption observed. | 3 | Optimize washing cycles; add water recycling unit. |
| Average Score |  |  | 2.5 / 5 | Transitioning from manual to semi-automated – early digital phase. |

| Process Area | Pillar | Observations | Score (1–5) | Comments / Recommendations |
| --- | --- | --- | --- | --- |
| Frying & Flavoring Process | Process Automation | Frying temperature and oil levels monitored manually. | 2 | Use smart temperature sensors & automatic oil level controllers. |
|  | Data Management | Batch records not digitized. | 2 | Integrate digital production logs with batch numbers. |
|  | Quality Monitoring | Visual quality check of wafers. | 2 | Introduce computer vision for color & crispness consistency. |
|  | Equipment Integration | Fryers and conveyors not connected digitally. | 2 | Add IoT modules for interconnectivity. |
|  | Workforce Skill | Operators experienced but lack exposure to smart devices. | 3 | Introduce training on data entry & dashboard usage. |
|  | Sustainability | High oil wastage; limited recycling. | 2 | Implement oil filtration and recycling system. |
| Average Score |  |  | 2.1 / 5 | Manual-heavy process — needs digitization focus. |